# A Dst-based space weather conditions machine learning classification model for GNSS PNT performance analysis

# Introduction

The Global Navigation Satellite System (GNSS) and its Positioning, Navigation, and Timing (PNT) service have matured to become an essential part of national infrastructure, public goods, and enablers of a vast number of emerging technology and socio-economic applications citestuff(filjar2022application). Maintaining the GNSS PNT quality has appeared fundamental for the sustainable development of modern economy and society citestuff(filjar2022application).

Overcoming the shortcomings and vulnerabilities of GNSS PNT is a scientific challenge, and the need of a wide variety of scientists, developers, operators, regulators, and users of GNSS-based systems and services citestuff(filjar2024ambient, filjar2022application). The Earth’s ionosphere, a part of the Earth’s atmosphere stretching from 50 km to 2000 km above the Earth’s surface and comprised of rare but mostly charged atoms and molecules, is the major natural cause of GNSS PNT degradation citestuff(davies1990ionospheric, filic2018modelling).

This phenomenon especially affects most currently used GNSS receivers, which work as single-frequency receivers exposed to GNSS ionospheric effects citestuff(spilker1996global, filjar2022application). Driven by powerful and random flows of energy and particles from the Sun (space weather conditions), the ionospheric conditions define the properties of GNSS signal propagation through the Earth’s atmosphere and the resulting ionospheric delay citestuff(davies1990ionospheric, oxley2017uncertainties, filic2018modelling). The GNSS ionospheric delay causes errors in GNSS PNT algorithm deployment, designed to produce position, velocity, and time estimates for a GNSS receiver citestuff(spilker1996global, filic2018modelling).

The ionosphere affects GNSS satellite signals for position estimation by introducing signal propagation delay citestuff(spilker1996global, filic2018modelling). The GNSS ionospheric delay is a stochastic variable, whose value is determined by complex physical processes of space weather citestuff(filic2018modelling, filjar2022application).

How space weather affects GNSS PNT performance was explained using the Space weather – GNSS PNT performance coupling model citestuff(filic2018modelling), as depicted in Table 1.

The Space weather – GNSS PNT performance coupling model, after citestuff(filic2018modelling).

| SPACE | GEOMAGNETIC | IONOSPHERE | GNSS | GNSS |
| --- | --- | --- | --- | --- |
| WEATHER | FIELD |  | PSEUDO- | POSITIONING |
|  |  |  | RANGES | PERFORMANCE |

The GNSS signal encounters a certain number of charged particles from the satellite aerial to a mobile unit’s (GNSS receiver’s) aerial citestuff(filic2018modelling, filic2018modelling). Such an encounter is quantified using the Total Electron Content (TEC) defined by Equation 1 in electrons/m2 citestuff(filic2018modelling, davies1990ionospheric), where h denotes the height above the Earth’s mean sea level in m, Nh represents the vertical ionospheric profile, the volume density of charged particles at height h, in electrons/m2.

TEC=lower ionsopheric boundaryupper ionsopheric boundaryNhdh 1

It should be noted that in the sense of Equation 1, TEC is defined as a result, a consequence, of the ionospheric conditions, and not their descriptor citestuff(filjar2022application). The GNSS ionospheric delay may be determined by derivation from the Appleton-Hartree equation, as given in Equation 2, where Δtiono denotes the GNSS ionospheric delay in s, c denotes the velocity of an electromagnetic wave in vacuum in m/s, and f denotes the carrier wave frequency of the satellite signal in Hz citestuff(spilker1996global, filic2018modelling).

Δtiono=40.3cf2lower ionsopheric boundaryupper ionsopheric boundaryNhdh 2

Combining Equation 1 and Equation 2, one can conclude the linear relation between Δtiono and TEC, as given in Equation 3, a sub-model incorporated in the Space weather – GNSS PNT performance coupling model citestuff(filic2018modelling).

Δtiono=40.3cf2TEC 3

The GNSS ionospheric delay has been identified as a source of GNSS PNT degradation since the dawn of GNSS. GNSS systems offer various standard GNSS ionospheric delay estimation (correction) models to mitigate the deteriorating effects on GNSS PNT, such as the Klobuchar model citestuff(spilker1996global, filic2018modelling). The standard ionospheric correction models are global, and insufficiently flexible to update to mitigate GNSS ionospheric delay to satisfy rising demands on GNSS PNT performance citestuff(filjar2024ambient, filic2018modelling). The development of regional and local models attempts to solve the problem of GNSS PNT sustainable performance in various ionospheric conditions citestuff(sikirica2021risk). In recent developments, our team has proposed the Ambient-Aware Application-Aligned ((AA)2) PNT to take into account the actual ionospheric and geomagnetic conditions near a mobile unit (a GNSS receiver) citestuff(filjar2022application, filic2018modelling, filjar2024ambient). Direct measurements of the immediate geomagnetic and ionospheric condition variables may be supplied to a machine learning-based adapted GNSS ionospheric correction model, thus solving the single-frequency GNSS problem citestuff(filjar2022application). Previous research has identified predictors and target variables (descriptors of geomagnetic, ionospheric, and GNSS PNT conditions) citestuff(filic2018modelling, natras2022ensemble, natras2023regional). The classification of different scenarios of GNSS ionospheric conditions with adverse effects on the GNSS PNT remained an unsolved precondition needed for the development of a machine learning-based GNSS ionospheric delay correction model to render the GNSS PNT algorithm ionospheric conditions-agnostic citestuff(filjar2022application, filjar2024ambient).

Here we propose a methodology for a machine learning-based classification of the ionospheric conditions based entirely on observations of geomagnetic indices. The proposed method is inspired by previous lightweight machine-learning techniques citestuff(filjar2020comparison) for classifying sensor readings. The model is sufficiently simple to be applied on computationally capable platforms with suitable geomagnetic field sensors, such as smartphones and connected/autonomous vehicles. The research presented acquires the ambient data and analyses its statistical properties. The data set is split into training and test sets. Several candidates for the GNSS ionospheric delay model are developed in this study based on Disturbance Storm-Time (Dst). The machine learning (ML) models include a Support Vector Machine (SVM) with a Polynomial Kernel, C5.0 Decision Tree (DT), Naive Bayes (NB), shallow Neural Network (NN), Partial Least Squares (PLS), Flexible Discriminant Analysis (FDA) and shallow Neural Network (NN) using Principal Component Analysis (PCA) of the input data. A tailored set of validation methods is used to assess their performance. The optimal GNSS ionospheric delay correction model is identified based on GNSS PNT-related objective criteria, and its performance is demonstrated in an independent case study.

The main contributions of this paper are:

- Defining multiple Dst-based classes using theoretical knowledge and statistics

- Classifying values of TEC and other geomagnetic field variables based on Dst

- Proposing a machine learning-based classification model of space weather events

- Achieving an accuracy over 99% for the Naive Bayes (NB) model and any input variables

The findings indicate the classification generated by the NB model could be used in real-life systems and support our hypothesis.

The rest of the paper is structured as follows. Section 2 describes this work’s methodology, dataset, and evaluation metrics. Section 3 presents the results obtained in this study. Section 4 discusses the implications of the results. The main points are summarised and the conclusion is given in Section 5.

# Method and Data

## Method

A Support Vector Machine (SVM) with a Polynomial Kernel, C5.0 Decision Tree (DT), Naive Bayes (NB), Neural Network (NN), Partial Least Squares (PLS), Flexible Discriminant Analysis (FDA) and shallow Neural Network (NN) using Principal Component Analysis (PCA) of the input data were tested based on their ability to classify a set of observations of the geomagnetic field in TEC, and other predictors, into one of the scenarios of space weather conditions based on Dst. Multiple Dst-dependent classes were predefined using theoretical knowledge. Statistical analysis of the data confirmed that distributions of other variables change for different Dst ranges, and not for different TEC ranges, supporting the validity of the classification. The study assumes that the dependent output variable, the Dst class, can be predicted based on the independent variables used as input.

The models were selected because they represent larger families of classification methods. SVM models are supervised maximum margin models. DT models also apply supervised learning. NB classifiers are probabilistic classifiers that can be parametric or non-parametric, but this study uses a non-parametric approach. PLS is a non-parametric linear regression model. FDA uses multiple non-parametric linear regression models to create a non-linear classification. PCA is a linear dimensionality reduction technique that extracts a predefined number of components for training an NN model. NN models imitate the brain using artificial neurons to produce outputs based on the input and the activation function. NN models require that the structure be predefined, and hyperparameters are usually tuned or predefined based on assumptions. An automated tuning procedure was used in this study.

All NN models were applied based on research by Kuhn for the R caret package citestuff(kuhn2013applied, KuhnCaret2024, rprojectProjectStatistical).

### Support Vector Machine

In machine learning, a Support Vector Machine (SVM) or Support Vector Network (SVN) model is a supervised maximum margin model with associated learning algorithms that analyzes data for classification and regression. In addition to linear classification, SVM models are also effective for non-linear classification using the hyperplane kernel trick citestuff(Boser1992), implicitly mapping their inputs to high-dimensional feature spaces.

An SVM model can also be used for regression tasks where the objective becomes ϵ -sensitive. A "soft margin" to separate data that is not linearly separable was proposed by Corinna Cortes and Vapnik in 1993 and published in 1995 citestuff(Cortes1995). Support Vector Clustering (SVC) citestuff(BenHur2001) applies support vector statistics to unlabeled data in unsupervised learning to find new natural groupings. Meyer, Leisch, and Hornik compared SVM with other classifiers citestuff(Meyer2003), but it is unclear whether SVM predictions perform better than other linear models, such as logistic, and linear regression.

A SVM constructs a set of hyperplanes in a high-dimensional space, used for classification, regression, or outlier detection citestuff(scikit-learn2023). Intuitively, a good separation is achieved by the hyperplane with the greatest distance to the nearest point in the training data belonging to any class citestuff(HastieRosset2009).

The original maximum margin hyperplane algorithm proposed by Vapnik in 1963 constructed a linear classifier. In 1992, Bernhard Boser, Isabelle Guyon, and Vladimir Vapnik proposed creating nonlinear classifiers using the kernel trick citestuff(Boser1992, Aizerman1964). A multidimensional feature space increases the generalization error of SVM models, so additional samples are needed to enhance performance citestuff(Jin2012). To keep the computational burden reasonable, a kernel probability density function kx,y is chosen to fit the problem citestuff(Press2007).

SVM models have been used to solve various real-world problems. SVM models are useful in text and hypertext categorization, as their application can significantly reduce the need for labeled training instances in standard inductive and transductive settings citestuff(Joachims1998). Some methods for shallow semantic parsing are based on SVM models citestuff(Pradhan2004). Image classification and segmentation can also be performed using SVM models, including a modified version of SVM that uses privileged access as proposed by Vapnik citestuff(Laurent2014, Barghout2015).

Classification of satellite data such as Synthetic Aperture Radar (SAR) data is possible using a supervised SVM citestuff(Maity2016). Handwritten characters can be recognized using SVM models citestuff(Decoste2002, Maitra2015). The SVM algorithm is widely used in biological and other sciences. They were used to classify proteins with up to 90% compounds correctly classified. Permutation tests based on SVM weights have been proposed as a mechanism for interpreting SVM models citestuff(Gaonkar2013, Cuingnet2011). SVM weights were also used to interpret SVM models citestuff(Statnikov2006).

The performance of SVM depends on the kernel probability density function, the parameters of the specified kernel probability density function, and the "soft" margin parameter λ. Each combination of parameter choices is usually cross-validated, and the parameters with the best accuracy are selected. The final model is used to test and classify new data and is trained using the best parameters citestuff(Hsu2003).

### Decision Tree

Decision Tree (DT) models are an approach to supervised learning in statistics, data mining, and machine learning. They represent predictive or decision-making model used to make inferences about a series of observations. Tree models in which the target variable can take on a discrete set of values are called classification trees. DT models where the target variable can take on continuous values (usually real numbers) are called regression trees. A regression tree can be extended to any object with pairwise differences such as categorical arrays citestuff(Studer2011). DT models are among the most popular ML algorithms due to their comprehensibility and simplicity citestuff(Wu2008). A DT describes the input data and the resulting classification tree can be the input for decision-making in data mining citestuff(Rokach2014).

A tree is built by dividing the original set, which forms the root node, into subsets that form descendants, or successors. The segmentation is made using a set of rules based on classification features citestuff(ShalevShwartz2014). This process is repeated on each derived subset in a recursive manner called recursive partitioning. The recursion is complete when the subset at a node all have the same target variable values or the division no longer adds value to the predictions. This process of Top-Down Induction of Decision Trees (TDIDT) citestuff(Quinlan1986) is an example of a greedy algorithm and is by far the most common strategy for learning DT models from data citestuff(Rokach2005). C5.0, used in the caret package in R, has a similar approach and improves the ID3 and C4.5 algorithms.

DT models can also be described as a combination of mathematical and computational techniques that help describe, categorize, and generalize a given data set in data mining. Data comes in records of the form x,Y=x1,x2,x3,…,xk,Y. The dependent variable Y is the target variable we need to understand, classify, or generalize. Features x1,x2,x3,…,xk form the vector X used for that task.

### Naive Bayes

In statistics, Naive Bayes (NB) models classifiers are a family of linear "probabilistic classifiers" that assume that, given a target class, the features are conditionally independent. The classifier got its name from this strong and possibly naive assumption. These classifiers are among the simplest Bayesian network models citestuff(McCallum2011). NB classifiers are highly scalable and require several parameters. The number of parameters depends linearly on the number of variables. Variables represent features or predictors in the learning problem. Maximum likelihood training can be performed by evaluating a closed-form expression citestuff(Russell1999), which requires linear time, instead of the expensive iterative approximation used for many other classifiers.

In the statistical literature, NB models are known by various names, including simple Bayes and independent Bayes citestuff(Hand2001). These names refer to Bayes’ theorem in the classifier’s decision rule, but a NB classifier model is not necessarily a Bayesian method citestuff(Russell1999, Hand2001). In other words, one can work with a NB model without accepting Bayesian probability or methods. In many practical applications, parameter estimation for NB models uses the maximum likelihood method.

Despite their simple design and seemingly oversimplified assumptions, NB classifier models have performed quite well in many complex real-world situations. In 2004, an analysis of the Bayesian classification problem showed that there are reasonable theoretical reasons for the seemingly incredible performance of NB classifier models citestuff(Zhang2004). However, a comprehensive comparison with other classification algorithms in 2006 showed that Bayesian classification performed worse than other approaches, such as boosted trees or random forests citestuff(Caruana2006). The advantage of NB models is a small amount of training data used to estimate the parameters needed for classification citestuff(stackexchange).

In the abstract, NB models are conditional probability models. The model assigns probabilities pCk∣x1,…,xn to each of the K possible outcomes or classes Ck given the problem instance to be classified, represented by the vector x=x1,…,xn which encodes n features representing mutually independent variables citestuff(Murty2011). The problem with the above formulation is that if the number of features n is large or the feature has a large value range, it is infeasible to base a model on probability tables. The model must therefore be reformulated using conditional probability represented by Bayes’ theorem in Equation 4.

pCk∣x=pCk px∣Ckpx 4

To estimate feature distribution parameters, we must assume a parametric distribution, or generate non-parametric models for the features from the training data set citestuff(John2013). Assumptions about the distribution of features are called an event model or Bayesian classifier, which can introduce confusion by creating both a continuous and discrete model citestuff(Mccallum2001, Metsis2006). Multinomial and Bernoulli distributions have often been used for discrete features in document classification, or spam filtering. The Gaussian NB classifier model was used since the features used in this study assumed continuous values.

Clustering is often used to discretize continuous values. The new set of features obtained by discretization follows the Bernoulli distribution. Some literature claims that discretization is necessary to use a NB classifier model, but discretization may discard the information needed to distinguish between classes citestuff(Hand2001).

Sometimes the distribution of class-based marginal densities is far from normal. The marginal probability density of each class can be used to estimate the kernel probability density function in such cases. This method, introduced by John and Langley citestuff(John2013), can significantly increase the accuracy of the classifier citestuff(Piryonesi2020, HastieRosset2009).

### Neural Networks

The neurons of human or animal brains provide the basis for a Neural Network (NN) or Artificial Neural Network (ANN) with connected units or nodes called artificial neurons in machine learning citestuff(mitExplainedNeural, brahme2014comprehensive).

Shallow NN models are simple and easy to train quickly with fewer parameters, and computational resources. They typically contain only a few hidden layers for processing between the input layer that receives the data, and the final layer that produces the output citestuff(olden2002illuminating, ozesmi1999artificial). A network with at least two hidden layers citestuff(bishop2006pattern) is considered a deep NN model.

NN models are typically trained through empirical risk minimization. This method optimizes the network’s parameters to minimize the difference or empirical risk, between the predicted output and the actual target values in a given dataset citestuff(vapnik2013nature). Gradient-based methods such as backpropagation are usually used to estimate network parameters citestuff(vapnik2013nature). During the training phase, ANN models learn from labeled training data by iteratively updating their parameters to minimize a defined loss function citestuff(goodfellow2016deep).

Using ANN models requires an understanding of their characteristics. The model suitable for a specific task varies depending on how the data is stored and other practical considerations. Many model parameters may need to be adjusted and differ for each model. The hyperparameters for the learning algorithm may also be modified to suit the problem citestuff(probst2019tunability) during an extensive tuning process experimenting on unseen training data, and various learning algorithms are available.

The network design process can be simplified using previously defined ML techniques like Neural Architecture Search (NAS). These networks have performed well even when pitted against architectures selected by humans. The proposed model is tested and the responses are used to train the NAS network citestuff(zoph2016neural). AutoML and AutoKeras citestuff(jin2019auto) provide extensive frameworks for this process. The scikit-learn library features various classification, regression, and clustering algorithms including SVM, Random Forest (RF), gradient boosting, k-means, and Density-Based Spatial Clustering (DBSCAN). The hyperparameters must be predefined, are not trained, and include how many neurons are in each layer, learning rate, step, stride, depth, receptive field, and padding for Convolutional Neural Network (CNN) citestuff(claesen2015hyperparameter). TensorFlow and Keras are used to build a custom network in Python, and the user can define layers, models, or metrics. The data, number of hidden layer units, learning rate, and number of iterations are required parameters for model training based on a book by Liu citestuff(liu2020python).

### Partial Least Squares

Partial Least Squares (PLS) regression, or projection to latent structures, citestuff(wold2001pls, abdi2010partial), is a linear regression statistical model that transforms the predicted and the observable variables to a new space instead of identifying maximum variance hyperplanes used in a linear regression model. Svante and Herman O. A. Wold, a Swedish father and son, founded this approach. PLS methods are bilinear factor models because the X and Y are projected to new spaces. In Partial Least Squares Discriminant Analysis (PLS-DA), Y is categorical citestuff(saebo2008lpls).

Using n paired observations xi,yi,i∈1,…,n. PLS finds the normalized direction pj,qj that maximizes the covariance in the first step j=1 citestuff(youtubeYouTube), shown in Equation 5. Matrix notation is used in further discussion of the model.

maxpj,qjE[pj⋅X⏟tjqj⋅Y⏟uj 5

The scores form an orthogonal basis in PLS regression, and the loadings are chosen to achieve this. In PCA orthogonality is not imposed on scores but onto loadings instead citestuff(lindgren1993kernel, de1994comments, dayal1997improved, de1993simpls, rannar1994pls, abdi2010partial). Many versions of PLS exist for estimating the factor and loading matrices, such as the PLS1 algorithm citestuff(takane2016pls, hoskuldsson1988pls).

### Flexible Discriminant Analysis

Flexible Discriminant Analysis (FDA) is a general methodology that creates the discriminant surface for a multigroup non-linear classification model based on a mixture of non-parametric linear regression models citestuff(hastie1995penalized), such as Multivariate Adaptive Regression Splines (MARS) and Linear Discriminant Analysis (LDA).

Many predictors can be used at once in FDA citestuff(HastieTibshirani2009), variable interactions are automatically noted citestuff(hastie1994flexible), it is complex but execution time and computational load are adequate citestuff(reynes2006choice). The algorithm is not significantly affected by outliers citestuff(phillips2017applying).

Modifying different settings has a high impact on FDA citestuff(hallgren2019species), and the estimation might be unsuccessful if the predictor distributions are highly correlated. FDA is prone to overfitting citestuff(thuiller2016ensemble), and challenging to comprehend or explain citestuff(quillfeldt2017influence). The assumption of normality for the continuous independent variables is necessary citestuff(zhang2019using) to determine the response or grouping categorical variable.

LDA, Normal Discriminant Analysis (NDA), or discriminant function analysis is a generalization of Fisher’s linear LDA defined in 1936 citestuff(cohen2013applied). The results of LDA may be utilized directly or to reduce dimensionality before classification, which is more often used citestuff(hansen2005using). LDA finds a linear combination of features or measurements differentiating two or more sample classes citestuff(fisher1936use, mclachlan2005discriminant). Discriminant analysis has continuous independent variables and a categorical dependent variable representing the groups citestuff(wetcher2011analyzing).

LDA is closely related to PCA and factor analysis since they identify linear functions of multiple variables that fit the input with the smallest error citestuff(martinez2001pca). Discriminant correspondence analysis citestuff(abdi2007discriminant, perriere2003use) is equivalent to LDA for categorical independent variables. Points in discriminant function analysis are rated based on quantitative predictors and a group measure citestuff(cokluk2008discriminant) to represent classification citestuff(venables2013modern).

The assumptions of Multivariate Analysis Of Variance (MANOVA) are also true for discriminant analysis. The analysis is highly affected by outliers so there must be fewer predictor variables than samples in the smallest class citestuff(cokluk2008discriminant). In every level of the grouping variable, independent variables must be normal to achieve multivariate normality citestuff(cokluk2008discriminant, hansen2005using). Box’s M statistic citestuff(hansen2005using) test whether homogeneity of variance/covariance is established and whether variances among group variables change for predictor levels.

Quadratic instead of linear discriminant analysis should be utilized if covariances are different citestuff(cokluk2008discriminant). The scores of one predictor variable should not depend on other predictor variables for any sample, and the samples should be chosen at random citestuff(cokluk2008discriminant, hansen2005using). Discriminant analysis can handle small deviations from these requirements citestuff(lachenbruch1979discriminant) and may be used even if multivariate normality is not achieved, such as for dichotomous variables citestuff(klecka1980discriminant).

The discriminant rules suggest that if x∈Rj, then x∈j for a group j, and Rj sets of sample space. The discriminant analysis method identifies regions of Rj that decrease errors of classification citestuff(Hardle2003AppliedMS). A discriminant score describes how well a discriminant differentiates classes. citestuff(archive765Discriminant).

The discriminant analysis eigenvalue represents the characteristic root of each function, indicating the quality of the group separation produced by the function. The function differentiates better for a larger eigenvalue citestuff(cokluk2008discriminant). Eigenvalues do not have an upper limit citestuff(cokluk2008discriminant, hansen2005using), so the results should not be taken at face value.

Even though it is not universally accepted, some suggest eigenvalues be utilized as effect size measures citestuff(hansen2005using). The percent of correctly classified data, such as the kappa value, can also be used as the effect size while accounting for random matches in classification citestuff(hansen2005using). Kappa is not biased for any particular class, whether performance is high or low, and instead normalizes across all classes citestuff(israel2006performance).

Fisher’s linear discriminant and LDA are sometimes considered synonyms. However, Fisher citestuff(fisher1936use) describes a slightly different discriminant, which does not make some of the assumptions of LDA such as a normal distribution or the equality of groups covariances. The application of the Fisher discriminant is expanded to find a subspace capturing all class variability citestuff(archive765Discriminant) for multiple groups, as suggested by C. R. Rao citestuff(rao1948utilization).

### Principal Component Analysis

Principal Component Analysis (PCA) is a linear dimensionality reduction technique with applications in exploratory data analysis, visualization, and data preprocessing. It is used in preprocessing before training a Neural Network in the pcaNNe method in the caret package in R citestuff(ripley2007pattern). The data is linearly transformed into a new coordinate system such that the directions (principal components) capturing the largest variation in the data can be easily identified. Many studies use the first two principal components to plot the data and visually identify clusters of closely related data points citestuff(jolliffe2016principal).

PCA is closely related to factor and Canonical Correlation Analysis (CCA). PCA is the simplest of the true eigenvector-based multivariate analyses. PCA defines a new orthogonal coordinate system that optimally describes the variance in a single dataset citestuff(barnett1987origins, hsu2012spectral, markopoulos2017efficient, chachlakis2019l1). CCA does the same for the cross-covariance between two datasets. Robust and L1-norm-based variants of standard PCA have also been proposed citestuff(chachlakis2019l1, markopoulos2014optimal, zhan2015robust, ke2005robust).

Karl Pearson citestuff(pearson1901liii) conceived of PCA in 1901 as a counterpart of the principal axis theorem in mechanics citestuff(stewart2019introduction). Harold Hotelling discovered the Hotelling transform in multivariate quality control in the 1930s without knowledge of the previous work citestuff(hotelling1933analysis, hotelling1992relations). Synonyms for PCA include the discrete Proper Orthogonal Decomposition (POD) citestuff(berkooz1993proper, karhunen1946spektraltheorie, loeve1977elementary, sirovich1987turbulence) in mechanical engineering and the Kosambi–Karhunen–Loève or Karhunen–Loève Theorem (KLT) citestuff(sapatnekar2011overcoming, ghoman2012pod, archiveKarhunenLoeveTransform, giambartolomei2016, mallat1999wavelet, tang1998texture) in signal processing. Singular Value Decomposition (SVD) of X, established in the final quarter of the 19th century citestuff(stewart1993early), is also similar to PCA. Eigenvalue Decomposition (EVD) of a matrix XTX in linear algebra shares the mathematical foundations of PCA citestuff(gloub1996matrix, hayden2002observations). Factor analysis differs in several key features citestuff(jolliffe2002principal), but has many aspects in common with PCA. Empirical Orthogonal Functions (EOF) citestuff(lorenz1956empirical) in meteorological science were developed by Lorenz in 1956 citestuff(lorenz1956empirical). The Eckart–Young theorem citestuff(eckart1936approximation) and quasiharmonic modes citestuff(dove1993introduction) are both connected to PCA. Empirical modal analysis in structural dynamics and spectral decomposition in noise and vibration also take a similar approach to PCA.

The data in PCA is converted to a new coordinate system using a scalar projection, an orthogonal linear transformation on a real inner product space. The first coordinate or principal component covers the largest portion of the variance, decreasing with each following component citestuff(jolliffe2002principal, bengio2013representation). The significance of the principal components can be checked with parametric bootstrap for a smaller dataset to help decide how many principal components to keep citestuff(forkman2019hypothesis).

## Data Description and Analysis

Many parameters can describe the Earth’s geomagnetic field, but this study focuses on those describing disturbances of the geomagnetic field, most importantly Kp, Ap, TEC, and Dst. The Kp index quantifies global geomagnetic activity by measuring disturbances in the horizontal component of Earth’s magnetic field. The Kp index is based on 3-hour-based K indices from 12 geomagnetic locations.

The K index is defined by Table2.

The K index.

| Geomagnetic | The intensity of a |
| --- | --- |
| observatory K index | geomagnetic storm |
| 0 - 1 | Quiet geomagnetic conditions |
| 2 - 4 | Unsettled |
| 5 | Minor storm |
| 6 | Large storm |
| 7 | Severe geomagnetic storm |
| 8 - 9 | Very intense (extreme) geomagnetic storm |

The K index is an integer in the range 0 to 9 (NGK lower scare is 500, PAF lower scale is 750), the largest range of geomagnetic disturbances ax and ay in the two horizontal components X and Y during a 3-hour UT interval.

The Ap index is a single value of magnetic activity for the entire day, the average of 8 K indexes.

Elevated TEC levels can increase signal delay which can cause positioning errors, especially critical for high-precision applications. TEC can be calculated as indicated in Equation 1, Equation 2, and Equation 3.

The Global Positioning System (GPS) Ionospheric delay on the radio ray path between satellite and receiver is defined in Equation 6 citestuff(spilker1996global) where Δt represents the GPS ionospheric delay, c is the velocity of light, and nl is the refraction index on the path l.

Δt=1csatellitereceivernl−ldl 6

The GPS ionospheric delay Δρ… related to distance measurements can be defined using Equation 7 citestuff(spilker1996global).

Δρ=1csatellitereceivernl−ldl 7

Appleton’s formula in Equation 8 reasonably approximates the relationship between the refraction index and the free electron density distribution citestuff(davies1990ionospheric, spilker1996global). Nh is the vertical distribution of the free electron density over a height h above the surface of the Earth, and f is the frequency of the radio signal.

nl=1+40.3Nhf2 8

The combination of Equation 6 and Equation 8 yields Equation 9 that directly describes the relationship between the free electron density profile and the GPS ionospheric delay citestuff(spilker1996global).

Δt=40.3cf2receiversatelliteNhdh 9

The TEC citestuff(spilker1996global) in Equation 10 is calculated using the integral on the right side of Equation 9.

TEC=receiversatelliteNhdh 10

Using equation Equation 10, Equation 9 can be written as shown in Equation 11.

Δt=40.3cf2TEC 11

Following the same analogy, the equivalent GPS ionospheric delay is calculated in Equation 12.

Δρ=40.3cf2TEC 12

Considering the pseudo-range measurements taken on both GPS transmitting frequencies (f1=1575.42 MHz and f2=1227.60 MHz) and using Equation 12, the relation between the TEC and GPS pseudo-range measurements can be established in Equation 13 citestuff(spilker1996global):

ρ1−ρ2=40.3TEC1f12−1f22 13

Equation 13 can be rewritten as in Equation 14, defining TEC. The actual value of the TEC can be computed utilizing a dual-frequency GPS receiver and Equation 14.

TEC=ρ1−ρ240.3f1f22f22−f12 14

Considering the imposed regulations for common GPS dual-frequency receiver use, a different method for estimating the TEC and GPS ionospheric delay is needed. A standard GPS ionospheric delay model, such as the Klobuchar model in Equation 15 citestuff(enge1994global,spilker1996global). A cosine-like daily dynamic of GPS ionospheric delay values is assumed in the Klobuchar model, with the daily maximum appearing around 14:00 in local time citestuff(klobuchar1987ionospheric).

Δt=Ftn+Acos2πt−t0P 15

The next relevant parameter is Dst, also known as the Dst index, a measure of geomagnetic storm intensity. Dst describes ring currents forming above the sub-equatorial region and affecting the ionospheric regions in mid-latitudes.

The Dst index is derived from measurements taken by a network of ground-based magnetometer stations near the magnetic equator, which continuously monitor the horizontal component of Earth’s magnetic field. To calculate the Dst index, variations in the horizontal magnetic field from these stations are averaged, and this average is then subtracted from a baseline value representing the quiet-time magnetic field. The resulting Dst value, expressed in nanoteslas (nT), measures the intensity of geomagnetic disturbances, with more negative values indicating stronger geomagnetic storms. The performance of GNSS is linked to many dynamic space weather conditions. Solar activity can induce various effects that degrade the accuracy, availability, and reliability of GNSS PNT. Incorporating parameters such as the Kp and Ap indexes, which provide global measures of geomagnetic activity, alongside TEC and the Dst index, allows for a more detailed assessment of the space environment and its potential effects on GNSS signals. In this paper, Ap, TEC, and Dst were used with ,Bx, By, and Bz to train machine-learning models.

To generate classes, samples were split into 5 ranges, P (positive, Dst from 15 to 50), N (normal, Dst from −20 to 15), R (recovery, Dst from −55 to −20), T (through, Dst from −85 to −55), and E (extreme, Dst from −120 to −85), based on Dst values derived from theoretical knowledge of different storm phases. The ranges are listed in Table 3.

Dst-based classification rules.

| Dst | Storm phase classification |
| --- | --- |
| 15…50 | positive phase (P) |
| −20…15 | normal (N) |
| −55…−20 | recovery phase (R) |
| −85…−55 | through (T) |
| −120…−85 | extreme (E) |

Figure 1 contains a heat map of the correlation between all variables used in this study. Blue represents a low correlation, while red represents a higher correlation. Swapping the row and column leads to the same combination of variables, so the matrix is symmetrical concerning the secondary diagonal. The values on the secondary diagonal equal 1 and can be disregarded because all variables are fully correlated with themselves. Besides these values on the secondary diagonal, the highest correlation of 0.78 is between dTEC and TEC, as expected based on previously outlined theoretical aspects of these variables. Bx and Dst have a correlation coefficient of 0.52, the second highest value not on the secondary diagonal. This is the only other value larger than 0.5 not on the secondary diagonal. A high correlation of Bx and Dst supports the hypothesis that we can predict Dst values, ranges, or classes using Bx. The third highest correlation coefficient not on the secondary diagonal is between Bx and By and equals 0.44. The properties and definitions of these variables explain this correlation.

A heat map of the correlation between all variables used in this study, where red represents a higher correlation, while blue represents a low correlation. All variables are fully correlated with themselves, so all values on the secondary diagonal equal 1. The matrix is symmetrical concerning the secondary diagonal because the same combination of correlated variables is achieved when swapping the row and column.

The box plots of all variables for different ranges of Dst values in Figure 2, without restriction on the range of TEC, and in Figure 3, where the TEC is less than 300 nT, demonstrate that the minimum, maximum, and arithmetic mean of Ap decrease for larger Dst values regardless of TEC ranges. The opposite is true for Bx, as indicated by a high correlation of Bx and Dst in Figure 1. By exhibits the same trend as Bx, but it is less prominent. Bz is the most stable variable with the smallest changes related to Dst when observing Bx, By, and Bz, but an increase for larger Dst values can also be observed. Similar conclusions can be reached from the scatter plots of all variables for different ranges of Dst in Figure 4.

The scatter plots of all variables for different ranges of TEC values in Figure 5 demonstrate that their distribution is not significantly dependent on TEC. This is why the box plots in Figure 2 and Figure 3 that only differ in TEC ranges look very similar. A minimal difference is also evident between the histograms of the relative frequency for all variables in Figure 6, without restriction on the range of TEC, and in Figure 7, where the TEC is less than 300 nT. Table 3 provides the minimum, 1st quartile, median, arithmetic mean, 3rd quartile, and maximum values for all variables, supporting the conclusions made based on Figure 2 and Figure 6.

Box plots of all variables without restriction on the range of TEC for different ranges of Dst values defining the class label used in this study.

Box plots of all variables when the TEC is less than 300 nT for different ranges of Dst values defining the class label used in this study.

Scatter plots of all variables when the TEC is less than 300 nT for different ranges of Dst values defining the class label used in this study.

Scatter plots of all variables for different ranges of TEC values without restriction.

Histograms of the relative frequency for all variables without restriction on the range of TEC.

Histograms of the relative frequency for all variables when the TEC is less than 300 nT.

The minimum, 1st quartile, median, arithmetic mean, 3rd quartile, and maximum values for all variables.

|  | TEC | dTEC | Bx | By | Bz |
| --- | --- | --- | --- | --- | --- |
| Min. | 1.00 | 0.01 | 35268 | 1900 | −29651 |
| 1st Qu. | 7.26 | 1.66 | 35400 | 1988 | −29623 |
| Median | 19.62 | 3.13 | 35410 | 2001 | −29620 |
| Mean | 40.00 | 15.88 | 35412 | 2001 | −29617 |
| 3rd Qu. | 34.23 | 6.49 | 35423 | 2015 | −29613 |
| Max. | 1288.00 | 997.00 | 35527 | 2092 | −29570 |

### Data Preprocessing

Data preprocessing is recommended to increase classification accuracy citestuff(Fan2008), so this step was also applied to the input data before training the models in this study. There are many ways to standardize data, such as minimum-maximum, normalization by decimal scaling, and Z-score citestuff(Mohamad2013). Subtracting the mean and dividing by the variance for each feature are commonly used for SVM models citestuff(Fennell2019) and other models tested in this study, so this approach was chosen.

The values scale and center were used in the code for this study in the preProcess parameter for the train function from the caret package developed for R. The option center subtracts the mean of each feature while center divides by the standard deviation.

### Training and Testing Dataset

The samples are divided into training and testing datasets as close as possible to a ratio of 80% for training and 20% for testing. During the division, the share of classes in the original data was taken into account, that is, the division was stratified by class so that an approximately equal ratio of classes was present in both the training data and the testing data, which is a feature of the createDataPartition function from the caret R library that was used citestuff(KuhnDataSplitting2024, hyndman2018forecasting, createDataPartition2024).

## Performance Metrics

### Confusion Matrix

The metrics and terminology used to evaluate the classifier performance were taken from the confusion matrix defined in the R function confusionMatrix in the caret library citestuff(kuhn2008building, altman1994diagnostic1, altman1994diagnostic2, velez2007balanced, confusionMatrix2024). The confusion matrix is suitable for use in multiclass classification, which is the goal of this project. A confusion matrix is a table where the number of rows and columns corresponds to the number of classes. The cells report the number of samples in each class classified into any class. This enables a more detailed analysis than by observing the share of correct classifications, i.e. accuracy. Accuracy will give results that can lead us to a wrong conclusion if the data set is unbalanced, that is, if the numbers of observations in different classes vary significantly, which was the case in this paper, because out of a total of 1597 samples, 1170 of them belonged to the largest N class, and 3 to the smallest E class.

If there are only two classes (Yes and No), the metrics are calculated using an example of a confusion matrix in Table 5.

A True Positive (TP) classification result correctly indicates that the sample belongs to the positive class.

A True Negative (TN) classification result correctly indicates that the sample belongs to the negative class.

A False Positive (FP) classification result or type I error, incorrectly indicates that the sample belongs to the positive class because it truly belongs to the negative class.

A False Negative (FN) classification result or type II error, incorrectly indicates that the sample belongs to the negative class because it truly belongs to the positive class.

The confusion matrix for a two-class problem.

|  | Reference |  |
| --- | --- | --- |
| Prediction | Yes | No |
| Yes | TP | FP |
| No | FN | TN |

Sensitivity, recall, hit rate, or True Positive Rate (TPR), is calculated as TP/TP+FN, that is, the proportion of samples that are correctly classified as positive among all samples that truly belong to the positive class. Higher TPR values indicate better results for the positive class.

Specificity, selectivity, or True Negative Rate (TNR), is calculated using the expression TN/TN+FP, that is, the proportion of samples that are correctly classified as negative among all samples that truly belong to the negative class. Higher TNR values indicate better results for the negative class.

Prevalence is calculated as TP+FN/TP+FP+FN+TN for the positive class, that is, the proportion of samples that truly belong to the positive class among all samples. Prevalence values other than an even split by the number of classes indicate a class imbalance.

Precision, or Positive Predictive Value (PPV), is calculated as TP/TP+FP for a two-class problem, i.e. the proportion of truly positive results among all samples classified as positive. Higher PPV values indicate better results for the positive class.

The Negative Predictive Value (NPV) is calculated as TN/TN+FN for a two-class problem, i.e. the share of truly negative results among all samples that are classified as negative. Higher NPV values indicate better results for the negative class.

The Detection Rate (DR) is calculated as TP/TP+FP+FN+TN for the positive class, that is, that is, the proportion of samples that are correctly classified as positive among all samples.

The Detection Prevalence (DP) is calculated as TP+FP/TP+FP+FN+TN for the positive class, that is, that is, the proportion of samples that are correctly or incorrectly classified as positive among all samples.

The accuracy (Acc) is calculated as TP+TN/TP+TN+FP+FN, that is, the proportion of samples that are correctly classified among all samples.

Since accuracy is not usable in the case of unbalanced classes, Balanced Accuracy (BA) is introduced, which is calculated as TPR+TNR/2 and is arithmetic mean TPR and TNR, which are respectively centered on the positive and the negative class separately.

Another metric that can be used instead of accuracy to consider each class separately is the F1 score, i.e. the harmonic mean of PPV and TPR, equal to 2×PPV×TPR/PPV+TPR=2×TP/2×TP+FP+FN.

If there are more than two classes, results are calculated for each class separately by viewing that class as the positive class and samples of all other classes as negative, a "one versus all" approach.

The unweighted Kappa statistic and a p-value from McNemar’s test are also computed. McNemar’s test produces NA values with sparse tables, such as the one in this experiment, so it was not included in the results.

A 95% confidence interval for the accuracy rate is computed using binom.test and a one-sided test citestuff(binom.test2024, clopper1934use, conover1999practical, hollander2013nonparametric) to see if the accuracy is significantly larger than the No Information Rate (NIR) which is the percentage of the largest class.

### The Binomial Test

The binomial test citestuff(howell1992statistical, graphpadGraphPadPrism) for two classes is an exact test of the statistical significance of deviations from a theoretically expected distribution.

The binomial test is often utilized to validate a hypothesis about the probability (π) of success H0:π=π0, where users define π0 between 0 and 1. The expected number of successes would be nπ0 if the null hypothesis H0 were correct. If k out of n attempts are successful, the binomial distribution formula defines the probability in Equation 16.

PrX=k=nkpk1−pn−k 16

The probability of an outcome equally or more extreme than actual occurrences is used to calculate the p-value. This is a simple operation for a one-tailed test.

Equation 17 contains the expression for the p-value when testing the hypothesis that π<π0. We can use a range from k to n to test π>π0.

p=i=0kPrX=i=i=0kniπ0i1−π0n−i 17

A binomial distribution isn’t symmetric if π0≠0.5, so calculating a p-value for a two-tailed test is not as simple. The p-value from the one-tailed test cannot be doubled to obtain the result since all events must be considered, including ones more extreme than previously seen. These extreme events are equally or less probable than X=k. Equation 18 denotes all such events. Equation 19 is used to compute the two-tailed p-value.

I={i:PrX=i≤PrX=k} 18

p=i∈I​PrX=i=i∈I​niπ0i1−π0n−i 19

### Cohen’s Kappa Coefficient

Cohen’s kappa coefficient (κ, lowercase Greek kappa) indicates inter- and intra-rater reliability for classification citestuff(mchugh2012interrater). As it accounts for random matches in ratings, it is a better metric than simply computing the percentage of agreement. Interpreting indices of agreement is not straightforward, and is still a point of contention among experts. Using disagreement instead of agreement in ratings might be simpler citestuff(pontius2011death). Galton was the first to use a statistic similar to Cohen’s kappa in 1892 citestuff(galton1892finger, smeeton1985early). Jacob Cohen formally described the kappa coefficient in 1960 in the journal "Educational and Psychological Measurement" citestuff(cohen1960coefficient).

Two raters group N items into C mutually exclusive classes, and Cohen’s kappa measures their agreement. Equation 20 defines κ, where po is the relative observed agreement among raters, and pe is the hypothetical probability of chance agreement. The observed data influences the computed probabilities of each observer randomly selecting each category.

κ≡po−pe1−pe=1−1−po1−pe 20

If the raters completely align then κ=1. When there is no agreement other than what would be expected by chance and is given by pe, κ=0. If κ statistic is negative citestuff(sim2005kappa) there is no match between the raters, which can reflect a real tendency of the raters to give differing ratings.

Equation 21 is derived from the construction in Equation 22 and holds for k categories with N observations to categorize if nk1 is the number of times rater i predicted category k. In these equations, pk12 is the estimated probability that both rater 1 and rater 2 will classify the same item as k, and pk1 is the estimated probability that rater 1 will classify an item as k. A similar claim can be made for rater 2.

pe=1N2k​nk1nk2 21

pe=k​pk12=ind.k​pk1pk2=k​nk1Nnk2N=1N2k​nk1nk2 22

If ratings are independent Equation 23, can be used.

pk=k​pk1pk2 23

Equation 24 is used to estimate the term pk1 by using the number of items classified as k by rater 1 (nk1) divided by the total items to classify (N).

pk1=nk1N 24

(and similarly for rater 2).

Equation 25 contains the Cohen’s Kappa formula citestuff(chicco2021matthews) for the traditional confusion matrix for binary classifications in Table 5.

κ=2×TP×TN−FN×FPTP+FP×FP+TN+TP+FN×FN+TN 25

In this scenario, Cohen’s Kappa equals the Heidke skill score introduced by Myrick Haskell Doolittle in 1888 citestuff(heidke1926berechnung, philosophical1887bulletin).

# Research Results

The accuracy, CI, NIR, p-value, and Kappa statistic for each model when using all variables (all), all variables except Dst (no Dst), all variables except Dst, TEC, and dTEC (no TEC), only Bx, By, and Bz (coord), only Bx, By, and Ap (x y Ap), only Bx, Bz, and Ap (x z Ap), or only By, Bz, and Ap (y z Ap) is displayed in Table 6, Table 7, Table 8, Table 9, Table 10, Table 11, and Table 12.

The accuracy, CI, NIR, p-value, and Kappa statistic for each model when using all variables as input.

| Model | Acc | 95% CI | NIR | p-value | Kappa |
| --- | --- | --- | --- | --- | --- |
| SVM Poly | 0.7672 | 0.7457,0.7877 | 0.7322 | 0.0007444 | 0.3607 |
| C5.0 DT | 1 | 0.9977,1 | 0.7322 | <2.2×10−16 | 1 |
| NB | 0.9912 | 0.9853,0.9952 | 0.7322 | <2.2×10−16 | 0.9787 |
| NN | 1 | 0.9977,1 | 0.7322 | <2.2×10−16 | 1 |
| PLS | 0.9086 | 0.8934,0.9223 | 0.7322 | <2.2×10−16 | 0.7489 |
| FDA | 0.9199 | 0.9055,0.9327 | 0.7322 | <2.2×10−16 | 0.8017 |
| PCA NN | 0.9975 | 0.9936,0.9993 | 0.7322 | <2.2×10−16 | 0.9939 |

The accuracy, CI, NIR, p-value, and Kappa statistic for each model when using all variables except Dst as input.

| Model | Acc | 95% CI | NIR | p-value | Kappa |
| --- | --- | --- | --- | --- | --- |
| SVM Poly | 0.7284 | 0.7059,0.7501 | 0.7322 | 0.6447 | 0.1137 |
| C5.0 DT | 0.8492 | 0.8307,0.8664 | 0.7322 | <2.2×10−16 | 0.6165 |
| NB | 0.9906 | 0.9846,0.9947 | 0.7322 | <2.2×10−16 | 0.9771 |
| NN | 1 | 0.9977,1 | 0.7322 | <2.2×10−16 | 1 |
| PLS | 0.7929 | 0.7722,0.8125 | 0.7322 | 1.104e−08 | 0.3635 |
| FDA | 0.8141 | 0.7942,0.8329 | 0.7322 | 9.859e−15 | 0.5064 |
| PCA NN | 0.8436 | 0.8248,0.861 | 0.7322 | <2.2×10−16 | 0.5868 |

The accuracy, CI, NIR, p-value, and Kappa statistic for each model when using all variables except Dst, TEC, and dTEC as input.

| Model | Acc | 95% CI | NIR | p-value | Kappa |
| --- | --- | --- | --- | --- | --- |
| SVM Poly | 0.7378 | 0.7155,0.7592 | 0.7322 | 0.3168 | 0.1066 |
| C5.0 DT | 0.8448 | 0.8261,0.8622 | 0.7322 | <2.2×10−16 | 0.5882 |
| NB | 1 | 0.9977,1 | 0.7322 | <2.2×10−16 | 1 |
| NN | 1 | 0.9977,1 | 0.7322 | <2.2×10−16 | 1 |
| PLS | 0.7922 | 0.7715,0.8119 | 0.7322 | 1.556e−08 | 0.3634 |
| FDA | 0.8166 | 0.7968,0.8353 | 0.7322 | 1.416e−15 | 0.5092 |
| PCA NN | 0.8379 | 0.8189,0.8557 | 0.7322 | <2.2×10−16 | 0.5647 |

The accuracy, CI, NIR, p-value, and Kappa statistic for each model when using only Bx, By, and Bz as input.

| Model | Acc | 95% CI | NIR | p-value | Kappa |
| --- | --- | --- | --- | --- | --- |
| SVM Poly | 0.7409 | 0.7187,0.7623 | 0.7322 | 0.2234 | 0.0887 |
| C5.0 DT | 0.826 | 0.8065,0.8443 | 0.7322 | <2.2×10−16 | 0.5208 |
| NB | 1 | 0.9977,1 | 0.7322 | <2.2×10−16 | 1 |
| NN | 1 | 0.9977,1 | 0.7322 | <2.2×10−16 | 1 |
| PLS | 0.7835 | 0.7625,0.8035 | 0.7322 | 1.284e−06 | 0.298 |
| FDA | 0.7979 | 0.7773,0.8173 | 0.7322 | 6.224e−10 | 0.4541 |
| PCA NN | 0.8254 | 0.8059,0.8437 | 0.7322 | <2.2×10−16 | 0.5242 |

The accuracy, CI, NIR, p-value, and Kappa statistic for each model when using only Bx, By, and Ap as input.

| Model | Acc | 95% CI | NIR | p-value | Kappa |
| --- | --- | --- | --- | --- | --- |
| SVM Poly | 0.7416 | 0.7193,0.7629 | 0.7322 | 0.2068 | 0.103 |
| C5.0 DT | 0.8292 | 0.8098,0.8473 | 0.7322 | <2.2×10−16 | 0.5386 |
| NB | 1 | 0.9977,1 | 0.7322 | <2.2×10−16 | 1 |
| NN | 1 | 0.9977,1 | 0.7322 | <2.2×10−16 | 1 |
| PLS | 0.781 | 0.7599,0.801 | 0.7322 | 3.978e−06 | 0.3153 |
| FDA | 0.8204 | 0.8007,0.8389 | 0.7322 | <2.2×10−16 | 0.5099 |
| PCA NN | 0.8367 | 0.8176,0.8545 | 0.7322 | <2.2×10−16 | 0.5579 |

The accuracy, CI, NIR, p-value, and Kappa statistic for each model when using only Bx, Bz, and Ap as input.

| Model | Acc | 95% CI | NIR | p-value | Kappa |
| --- | --- | --- | --- | --- | --- |
| SVM Poly | 0.739 | 0.7168,0.7604 | 0.7322 | 0.2775 | 0.1013 |
| C5.0 DT | 0.8104 | 0.7903,0.8293 | 0.7322 | 1.602e−13 | 0.4714 |
| NB | 1 | 0.9977,1 | 0.7322 | <2.2×10−16 | 1 |
| NN | 1 | 0.9977,1 | 0.7322 | <2.2×10−16 | 1 |
| PLS | 0.7735 | 0.7521,0.7938 | 0.7322 | 8.468e−05 | 0.2742 |
| FDA | 0.7979 | 0.7773,0.8173 | 0.7322 | 6.224e−10 | 0.4343 |
| PCA NN | 0.8148 | 0.7948,0.8335 | 0.7322 | 6.107e−15 | 0.4948 |

The accuracy, CI, NIR, p-value, and Kappa statistic for each model when using only By, Bz, and Ap as input.

| Model | Acc | 95% CI | NIR | p-value | Kappa |
| --- | --- | --- | --- | --- | --- |
| SVM Poly | 0.7347 | 0.7123,0.7562 | 0.7322 | 0.4233 | 0.0251 |
| C5.0 DT | 0.7866 | 0.7657,0.8065 | 0.7322 | 2.88e−07 | 0.3926 |
| NB | 0.9987 | 0.9955,0.9998 | 0.7322 | <2.2×10−16 | 0.9969 |
| NN | 1 | 0.9977,1 | 0.7322 | <2.2×10−16 | 1 |
| PLS | 0.7566 | 0.7348,0.7774 | 0.7322 | 0.01419 | 0.2075 |
| FDA | 0.7672 | 0.7457,0.7877 | 0.7322 | 0.0007444 | 0.3367 |
| PCA NN | 0.7797 | 0.7586,0.7998 | 0.7322 | 6.857e−06 | 0.3878 |

The execution time in s is calculated using the R system.time function citestuff(system.time2024) for each model when using different combinations of variables as input is displayed in Table 13. The experiment was run on Windows 11 using R Studio version 2024.04.2+764 and R version 4.4.1, the AMD Radeon RX 6600 Graphics Processing Unit (GPU), 16 GB of Random Access Memory (RAM), and the AMD Ryzen 5 PRO 4650G Central Processing Unit (CPU) with 6 cores.

The execution time in seconds for each model when using different combinations of variables as input.

| Model | SVM Poly | C5.0 DT | NB | NN | PLS | FDA | pcaNNet |
| --- | --- | --- | --- | --- | --- | --- | --- |
| all | 103.52 | 169.28 | 192.05 | 494.53 | 49.99 | 51.57 | 291.98 |
| no Dst | 276.24 | 592.32 | 175.38 | 479.67 | 42.89 | 49.34 | 305.16 |
| no TEC | 285.58 | 393.72 | 137.99 | 453.82 | 33.11 | 60.53 | 294.58 |
| coord | 331.13 | 277.63 | 118.86 | 441.37 | 22.28 | 67.94 | 294.97 |
| x y Ap | 319.97 | 295.20 | 121.73 | 465.61 | 26.72 | 70.97 | 302.48 |
| x z Ap | 348.47 | 259.81 | 121.05 | 475.05 | 25.56 | 76.04 | 321.05 |
| y z Ap | 416.75 | 277.62 | 124.02 | 489.08 | 29.89 | 95.64 | 345.05 |

## Comprehensive Results for the Naive Bayes Model

The confusion matrix and the performance indicators derived from it when using the Naive Bayes (NB) model and all input variables are depicted in Table 14 and Table 15. The same values when using all input variables except Dst are marked in Table 16 and Table 17, and Table 18 and Table 19 when using only By, Bz, and Ap variables as input. These results are displayed since the NB model has low computational load and high accuracy. The combinations of input variables that produced a 100% accuracy when used for the NB model were not included since there are no errors in classification.

The confusion matrix for the Naive Bayes model when using all variables as input.

|  | Reference |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Prediction | E | N | P | R | T |
| E | 3 | 1 | 0 | 0 | 0 |
| N | 0 | 1161 | 1 | 1 | 0 |
| P | 0 | 5 | 26 | 2 | 0 |
| R | 0 | 3 | 0 | 375 | 0 |
| T | 0 | 0 | 0 | 1 | 19 |

The performance indicators derived from the confusion matrix for the Naive Bayes model when using all variables as input.

|  | Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Statistics | E | N | P | R | T |
| Sensitivity | 100% | 99.23% | 96.296% | 98.94% | 100% |
| Specificity | 99.9373% | 99.53% | 99.554% | 99.75% | 99.937% |
| PPV | 75% | 99.83% | 78.788% | 99.21% | 95% |
| NPV | 100% | 97.93% | 99.936% | 99.67% | 100% |
| DR | 0.1877% | 72.65% | 1.627% | 23.47% | 1.189% |
| DP | 0.2503% | 72.78% | 2.065% | 23.65% | 1.252% |
| BA | 99.9687% | 99.38% | 97.925% | 99.35% | 99.968% |

The confusion matrix for the Naive Bayes model when using all variables except Dst as input.

|  | Reference |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Prediction | E | N | P | R | T |
| E | 3 | 1 | 0 | 0 | 0 |
| N | 0 | 1161 | 2 | 2 | 0 |
| P | 0 | 5 | 25 | 1 | 0 |
| R | 0 | 3 | 0 | 375 | 0 |
| T | 0 | 0 | 0 | 1 | 19 |

The performance indicators derived from the confusion matrix for the Naive Bayes model when using all variables except Dst as input.

|  | Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Statistics | E | N | P | R | T |
| Sensitivity | 100% | 99.23% | 92.593% | 98.94% | 100% |
| Specificity | 99.9373% | 99.07% | 99.618% | 99.75% | 99.937% |
| PPV | 75% | 99.66% | 80.645% | 99.21% | 95% |
| NPV | 100% | 97.92% | 99.872% | 99.67% | 100% |
| DR | 0.1877% | 72.65% | 1.564% | 23.47% | 1.189% |
| DP | 0.2503% | 72.9% | 1.94% | 23.65% | 1.252% |
| BA | 99.9687% | 99.15% | 96.105% | 99.35% | 99.968% |

The confusion matrix for the Naive Bayes model when using only By, Bz, and Ap as input.

|  | Reference |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Prediction | E | N | P | R | T |
| E | 2 | 0 | 0 | 0 | 0 |
| N | 1 | 1170 | 1 | 0 | 0 |
| P | 0 | 0 | 26 | 0 | 0 |
| R | 0 | 0 | 0 | 379 | 0 |
| T | 0 | 0 | 0 | 0 | 19 |

The performance indicators derived from the confusion matrix for the Naive Bayes model when using only By, Bz, and Ap as input.

|  | Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Statistics | E | N | P | R | T |
| Sensitivity | 66.6667% | 100% | 96.296% | 100% | 100% |
| Specificity | 100% | 99.53% | 100% | 100% | 100% |
| PPV | 100% | 99.83% | 100% | 100% | 100% |
| NPV | 99.9373% | 100% | 99.936% | 100% | 100% |
| DR | 0.1252% | 73.22% | 1.627% | 23.72% | 1.189% |
| DP | 0.1252% | 73.34% | 1.627% | 23.72% | 1.189% |
| BA | 83.3333% | 99.77% | 98.148% | 100% | 100% |

# Discussion

The Neural Network (NN) model has the highest execution time for any subset of the input variables due to its complexity and extensive training, evident from the data in Table 13. However, in each case, it achieved a 100% accuracy, which is shown in Table 6, Table 7, Table 8, Table 9, Table 10, Table 11, and Table 12. The Naive Bayes (NB) model has the second highest accuracy, over 99%. The NB model achieved a 100% accuracy, except when using all input variables, all input variables except Disturbance Storm-Time (Dst), or only By, Bz, and Ap. The difference in accuracy between the NN and the NB model is negligible, and the training time for the NN model is more than twice as long. Despite their simple design and seemingly oversimplified assumptions, NB classifiers have performed quite well in many complex real-world situations. In 2004, an analysis of the Bayesian classification problem showed that there are reasonable theoretical reasons for the seemingly incredible performance of NB classifiers citestuff(Zhang2004).

All other models fail to achieve an accuracy over 90%, except when using all input variables, including Dst. The Dst class was derived by thresholding the continuous Dst value and discretizing it by converting it to a single character, so the original Dst value should not be used as model input. This labeling method explains the increase in accuracy when adding the Dst input variable in all except the two best-performing models. For example, the C5.0 DT model did not consider any variable except Dst, indicated by a 100% variable importance. The Support Vector Machine (SVM) model with a Polynomial Kernel is consistently the worst-performing for any subset of input variables, never achieving an accuracy over 80%. None of the models achieved an accuracy under 70%, so they were all moderately successful.

When studying the performance of the NB model by individual class in Table 14, Table 15, Table 16, Table 17, Table 18 and Table 19, samples of the T class are all correctly classified, but the testing is less extensive since it is the second smallest class. Samples of the R class are sometimes erroneously assigned to the N, P, or less commonly the T class. Samples of the P class are rarely mistaken for the N class. Two of the most common misclassifications were assigning samples of the N class with the highest detection rate and prevalence to the P or R classes with the third and second highest detection rate and prevalence. This is due to the largest number of samples in the N class between the P and R classes with the narrowest Dst range. Samples of the N class are least often included in the E class, whose range is the furthest apart from the N class. A sample of the E class was attributed to the N class only once. The E class is the smallest with only three samples, which must be accounted for when interpreting these results, such as the lowest balanced accuracy of 83.33% and sensitivity of 66.67% when using only By, Bz, and Ap as input, or the lowest positive predictive value of 75% when using all variables as input.

# Conclusion

The presented study aims to classify ambient conditions of space weather events for sub-equatorial regions. Global Navigation Satellite System (GNSS) Positioning, Navigation, and Timing (PNT) performance is significantly affected by such events. it would be beneficial to indicate to users that a geomagnetic/ionospheric storm is in progress.

Classification models using machine learning were applied to descriptions of the geomagnetic field expressed in Total Electron Content (TEC) and other input data. It was assumed observations contained independent variables to generate the dependent variable representing the Disturbance Storm-Time (Dst) class.

Statistical analysis confirmed that other variables change distribution based on Dst, not TEC. Continuous Dst values in different ranges were converted into discrete classes based on statistics, previous theories, and research.

An Support Vector Machine (SVM) with a Polynomial Kernel, C5.0 Decision Tree (DT), Naive Bayes (NB), Neural Network (NN), Partial Least Squares (PLS), Flexible Discriminant Analysis (FDA), and Principal Component Analysis (PCA) NN model created a Dst-based classification from multiple combinations of input variables.

The NB model achieved perfect accuracy for most tested combinations of input variables, and over 99% in all cases. It is slightly less successful than the NN model, and the total execution time is at least two times shorter, making it more suitable for use in compact, low-performance, and low-cost portable devices such as smartphones.

# Acronyms

(AA)2 Ambient-Aware Application-Aligned

ANN Artificial Neural Network

BA Balanced Accuracy

CCA Canonical Correlation Analysis CI Confidence Interval CNN Convolutional Neural Network CPU Central Processing Unit

DBSCAN Density-Based Spatial Clustering DP Detection Prevalence DR Detection Rate DT Decision Tree Dst Disturbance Storm-Time

EOF Empirical Orthogonal Functions EVD Eigenvalue Decomposition

FDA Flexible Discriminant Analysis FN False Negative FP False Positive

GNSS Global Navigation Satellite System GPS Global Positioning System GPU Graphics Processing Unit

KLT Karhunen–Loève Theorem

LDA Linear Discriminant Analysis

MANOVA Multivariate Analysis Of Variance MARS Multivariate Adaptive Regression Splines ML machine learning

NAS Neural Architecture Search NB Naive Bayes NDA Normal Discriminant Analysis NIR No Information Rate NN Neural Network NPV Negative Predictive Value

PCA Principal Component Analysis PLS Partial Least Squares PLS-DA Partial Least Squares Discriminant Analysis PNT Positioning, Navigation, and Timing POD Proper Orthogonal Decomposition PPV Positive Predictive Value

RAM Random Access Memory RF Random Forest

SAR Synthetic Aperture Radar SVC Support Vector Clustering SVD Singular Value Decomposition SVM Support Vector Machine SVN Support Vector Network

TDIDT Top-Down Induction of Decision Trees TEC Total Electron Content TN True Negative TNR True Negative Rate TP True Positive TPR True Positive Rate

# Declarations

## Availability of data and materials

The datasets used during the current study are available from the corresponding author upon reasonable request.

## Competing interests

The authors declare no conflict of interest.

## Funding

This work was fully supported by

## Authors’ contributions

Lucija Žužić: Conceptualization, Methodology, Software, Validation, Investigation, Data Curation, Writing – Original Draft, Visualization.

Teodor B. Iliev: Conceptualization, Methodology, Validation, Formal analysis, Investigation, Resources, Writing – Original Draft, Writing – Review & Editing, Supervision, Project administration, Funding acquisition.

Renato Filjar: Conceptualization, Methodology, Validation, Formal analysis, Investigation, Resources, Writing – Original Draft, Writing – Review & Editing, Supervision, Project administration, Funding acquisition.

## Acknowledgements

This work was fully supported by

## Authors’ information

Lucija Žužić is a doctoral student at the Faculty of Engineering, University of Rijeka, currently employed as an assistant at the Department of Computer Engineering. Her research interests include applied machine learning in biology, chemistry, medicine, and transportation. Maritime transportation is also the subject of her master’s thesis.

Teodor B. Iliev

Renato Filjar